INKPAD RESETS

（Ink lamp and paper empty lamp flash alternately）

1   AdjProg

1   Open the maintenance software AdjProg. Choose Accept

2   Particular adjustment mode

2   select  Particular adjustment mode

3 Waste ink pad counter

3  Drop down menu selects the waste ink pad counter

4 Main pad counter, Platen pad counter, Initialization

4 Check Main pad countre and Platen pad counter，then click Initialization

5 When the dialog box pops up, choose . There will be a progress bar at this time.

6 After the progress bar is finished, we will prompt us to turn off the printer power (refer to the printer power button, not the total power of the machine)

When the printer is turned off, the power light no longer flashes, and then click .

Button icon

7 At this time, we will prompt us to re-power the printer and confirm that it is complete.